# TASK 2

# 1. High Grade Material Flow

| Step | Location/Equipment |
| --- | --- |
| Source | P1-Pit (High Grade) |
| Transport | ↓ Truck (777G) |
| Stockpile | ↓ Finger 1 (Stockpile) |
| Rehandle | ↓ Loader (992K) |
| Destination | Main Crusher |

# -----2. Low Grade Material Flow

| Step | Location/Equipment |
| --- | --- |
| Source | P2-Pit (Low Grade) |
| Transport | ↓ Truck (777G) |
| Stockpile | ↓ Finger 2 (Stockpile) |
| Rehandle | ↓ Loader (992K) |
| Destination | Main Crusher |

# Mining Dispatch Data Investigation Report

Client: Integrated Mining Platform (IMP)
Duration Analyzed: 15 June 2025 – 13 July 2025
Datasets Used:

- truck_dump_logs.csv

- rehandle_logs.csv

## 1 Objective

The client suspects ore dilution and volume mismatch between the Pit and the Main Crusher.
The goal of this analysis was to:

- Map the material flow through the site

- Find evidence supporting dilution or volume mismatch

- Propose one key metric for a dashboard to prevent this issue

## 2️ Material Flow Mapping

Based on loading_location and dumping_location columns, the physical material flow is:

P1-Pit (High Grade) → Finger 1 → Main Crusher

P2-Pit (Low Grade)  → Finger 2 → Main Crusher

Equipment involved:

- Trucks (777G): Pit → Stockpiles (Fingers)

- Loaders (992K): Stockpiles → Main Crusher

### Flow Deviation Observed

Truck dump logs show misrouting:

- High Grade material dumped into Finger 2

- Low Grade material dumped into Finger 1

This indicates stockpile-level contamination risk.

## 3️ Investigation of Client Complaint

### Volume Reconciliation (Daily Basis)

Daily volumes were aggregated and compared:

- Pit → Finger (Truck logs): Total mined tons

- Finger → Crusher (Rehandle logs): Total processed tons

Recovery % formula:

Recovery % = (Crusher Tons / Pit Tons) × 100

### Key Findings

| Finger | Material | Typical Daily Recovery |
| --- | --- | --- |
| Finger 1 | High Grade | ~8–9% |
| Finger 2 | Low Grade | ~12–15% |

More than 85–90% of material does not reach the crusher on the same day, strongly confirming a volume mismatch.

## 4 Dilution Root Cause Analysis

A check was performed to see if loaders were mixing materials on the same day:

groupby(loading_location, shift_date).material.nunique()

### Result

- Material count = 1 for all Fingers on all days

- No same-day material mixing during rehandle

### Conclusion

- Rehandle process is clean

- Dilution and mismatch originate upstream, during truck dumping and stockpile entry

## 5️ Lead Pitch: Preventive Dashboard Metric

### Recommended Single KPI

## Daily Ore Recovery % (Pit → Crusher)

Why this metric?

- Directly highlights volume mismatch

- Easy to understand for operations and management

- Can be segmented by:

  - Material type

  - Stockpile (Finger)

  - Date

Alert Threshold:

Recovery % < 90% → Investigation required